CV Quality Inspection — Toy PoC vs. Real Deployment

What we're building this week, and how it maps to the real Jetson-based system

# 1. What we're doing right now

Goal: build a small proof-of-concept that validates the sense → think → verdict pipeline concept, before touching any of the real hardware (Jetson, PLC, OPC-UA).

Scope: only the “Think” stage — training and testing a computer vision model that looks at an image of an object and decides good vs. defective. No barcode scanner, no PLC, no actuation.

## Approach

- PatchCore anomaly detection — a frozen ImageNet-pretrained ResNet-18 turns each image into a grid of local patch features. No weights are trained.

- Trained only on “good” images per object category — no defect images needed for training.

- No gradient training loop: the patch features from every “good” image are collected into a memory bank, then compressed with a greedy coreset (a representative subset). That memory bank is the model.

- At test time: each test patch is scored by its distance to the nearest “good” patch in the bank; a defect sits far from anything normal → per-patch anomaly map (heatmap), image score = worst patch → threshold → ACCEPT / REJECT.

## Objects

- Bottle, screw, tile, toothbrush — 4 categories from the MVTec AD dataset for training and metrics.

- Physical versions of the same objects, captured on a laptop webcam, for a live demo.

## Note on the live webcam demo

The model is trained on MVTec AD images captured under controlled conditions; the physical demo objects are viewed through a laptop webcam, so some domain shift is expected. PoC success is measured on the MVTec test sets — the webcam demo is an illustration on top of that.

## Why one-class (not a normal binary classifier)

A binary good/bad classifier needs a large, balanced set of labeled defect images, and only recognizes defect types it was explicitly trained on. Defects on a real line are rare, varied, and expensive to collect — new defect types show up that were never in the training set. Training only on “good” avoids this: the model just needs to learn what normal looks like, and flags anything sufficiently different — including defect types it has never seen. This is also why the real system uses this same one-class philosophy (via PatchCore).

## Deliverables (by Monday)

- Jupyter notebook: data loading, model, training, evaluation (AUROC per category, plus precision/recall at the operating threshold, heatmaps).

- Live webcam inference script.

- Short written report: approach, results, limitations, and path to real deployment.

## What counts as success

This PoC is considered successful if:

- AUROC ≥ 0.85 per category on the MVTec AD test sets (defect images used for evaluation only, never training).

- Heatmaps localize the defective region on MVTec test images.

If these hold, the sense → think → verdict logic is validated and ready to carry into the Jetson phase.

# 2. Side-by-side: PoC vs. real system

| Stage | Toy PoC (this week) | Real deployment (Jetson phase) |
| --- | --- | --- |
| Camera | MacBook built-in webcam | Basler ace2 GigE industrial camera, triggered by barcode scan |
| Objects | Bottle, screw, tile, toothbrush (MVTec AD + physical samples) | Actual automotive parts on the production line (e.g. Honda Ohio) |
| Model | PatchCore — frozen ImageNet ResNet-18 backbone + greedy-coreset memory bank of “good” patches, one per object category (no gradient training) | Same PatchCore architecture carried forward, likely a stronger backbone (e.g. WideResNet-50) + memory bank, calibrated on in-domain “good” images from the line |
| Where it runs | MacBook Air M3 (CPU / Apple MPS) | NVIDIA Jetson Orin (GPU), model converted to TensorRT for fast on-device inference |
| Training location | Local laptop, this week | Off-device (dev machine / cloud), before deployment — Jetson never trains, only runs inference |
| Verdict output | Printed to console / shown in notebook (ACCEPT / REJECT + heatmap) | Sent via OPC-UA to the Siemens S7-1500 PLC |
| Actuation | None — no physical action taken | PLC fires the actuator (arm, air jet, gate, etc.) to physically remove rejected parts |
| Goal | Prove the sense → think → verdict concept works end to end | Run that same logic reliably, in real time, at production line speed |

## What this PoC does not prove

- Real-time performance: no latency or throughput testing at line speed — that is what the TensorRT/Jetson phase validates.

- Hardware integration: no camera triggering, PLC communication, or actuation — the OPC-UA path is exercised only in the real deployment.

- Production imaging conditions: MVTec and webcam images do not replicate the Basler camera’s optics and lighting on the actual line.

The PoC validates the detection logic; the Jetson phase validates that the same logic runs reliably at production speed.

# 3. Where NVIDIA / Jetson actually comes in

Jetson is a deployment target, not a training environment. It runs inference only — it never trains a model.

The real flow, once this PoC validates the concept:

- 1. Design and train the model on a dev machine (this is the part we're doing now, just with a laptop and simpler tooling).

- 2. Convert the trained model to TensorRT — NVIDIA's optimized inference format for their GPUs, so it runs fast enough for real-time inspection on the line.

- 3. Deploy the converted model onto the Jetson Orin, which processes live frames from the Basler camera as parts pass by.

- 4. Jetson outputs just the verdict (accept/reject) — no PLC logic, no actuation decision — and sends it via OPC-UA.

- 5. The Siemens S7-1500 PLC receives that verdict and physically fires the actuator.

In short: everything we build this week — the model architecture, the training approach, the scoring/thresholding logic — is the same logic Jetson will eventually run. The PoC swaps out only the hardware and communication layer (webcam → industrial camera, console print → OPC-UA, MacBook → Jetson+TensorRT, no actuation → PLC actuation).